DHT + BLYNK

IoT Bootcamp Assignment 10
Presented by: Smitha Naik

Connections / Blynk: DHT11 DATA → GPIO 4; Blynk V1 → temperature; V2 → humidity. Install DHT sensor library by Adafruit.

Arduino Code

#define BLYNK_TEMPLATE_ID "TMPL3NDg3-dys"
#define BLYNK_TEMPLATE_NAME "DHT"
#define BLYNK_AUTH_TOKEN "2MeyHhRIeEHiGHQvpsbWY1slcdFzG-Br"

#include <WiFi.h>
#include <BlynkSimpleEsp32.h>
#include <DHT.h>

char ssid[] = "AirFiber-Uqu3AM";
char pass[] = "12346360";

#define DHTPIN 4
#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup()
{
  Serial.begin(115200);
  dht.begin();
  Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);
}

void loop()
{
  Blynk.run();

  float temperature = dht.readTemperature();
  float humidity = dht.readHumidity();

  Serial.print("Temperature: ");
  Serial.print(temperature);
  Serial.println(" C");

  Serial.print("Humidity: ");
  Serial.print(humidity);
  Serial.println(" %");

  Blynk.virtualWrite(V1, temperature);
  Blynk.virtualWrite(V2, humidity);

  delay(2000);
}

Note: Keep your Blynk Auth Token and Wi-Fi password private.